MATERIAL SAFETY DATA SHEET

Section I

| Manufacturer's Name : Cessco | Emergency Telephone Number | (704) 375-9824 |
| --- | --- | --- |
| Address (Number. Sfreet. City, State, and ZIP Code; 1109-1101 Central Avenue, PO 16452 | Telephone Number tor Intormatk | (800) 4-cressco |
| Charlotte, NC 28216 | Data Prepared 08/01/89 |  |
|  | Chemical Name: Dursban Chemical Family: Organophosphorous Compound |  |

Section II — Product and Component Data

Cornponent (s)    Chemical Name                  Registration No.                           OSHA PEL

O,O – Diethyl O – (3,5,6 – trichloro-2-pyridinyl

Phosporothioate (chlorpyrifos  CAS 2921-88-2

Petroleum Oil, Refined  CAS 64742-88-7

1,1,1-Trichloroethane  CAS 70-55-6

Chlorodifluoromethane  CAS 75-45-6

% Approx

1.00

19.0

40.0

40.0

TLV Data

3.2mg/m3

100 ppm

200 ppm

3,000 ppm

Section III — Physical Data

| Boiling Point | NA | Specific Gravity (H2O = 1) | 0.164 |
| --- | --- | --- | --- |
| Vapor Pressure (mm Hg.) | 3878 | Percent Volatile | >97.5 |
| Vapor Density (AIR - 1) | NA | Evaporation Rate (Butyl Acetate - t) | - |
| Solubility in Water | NA | Appearance and Odor |  |

.

Section IV — Fire and Explosion Hazard Data

| Flash Point (Method Used) | NA | Flammable Limits | LEL | UEL |
| --- | --- | --- | --- | --- |

Extinguishing Media

CO2, Dry Chemical, Foam.

Special Fire Fighting Procedures

Water positive pressure self-contained breathing apparatus.  Keep containers cool to avoid bursting.

Unusual Fire and Explosion Hazards

Exposure to temperatures above 130 degrees F may cause bursting.  Irritating gasses.  Product decomposes rapidly at + 160 c.

Section V – Health Hazard Data

Threshold Limit Value – approximately 18-20 mg/M3

Effects of Overexposure – headaches, dizziness, constrictel pupils, nausea, vomiting

Emergency and First Aid Procedures  - eyes:  irrigate with water for 15 minutes.  Skin: remove contaminated clothing and rinse with water for 15 minutes.  Ingestion: DO NOT induce vomiting, drink several glasses of water or milk.  Inhalation:  remove to fresh air.  Consult a physician.

Section VI —Reactivity Data

| Stability | Unstable |  | Conditions to avoid: At high temperatures, product goes exothermic decomposition which can lead to higher temp. and violent decomposition |
| --- | --- | --- | --- |
|  | Stable | X |  |

Incompatability (Materials to Avoid)

Base Materials

| Hazardous Decomposition or Byproducts Sulfurdioxide, hydrogen, chloride, organic sulfur compounds, hydrofluoric and hydrochloric acids |  |  |  |
| --- | --- | --- | --- |
| Hazardous Polymerization | May Occur |  | Conditions to Avoid |
|  | Will Not Occur | X |  |

Section VII – Spill or Leak Procedures

Steps to be taken in case material is released or spilled:  If container is ruptured or begins to lead, place in a well ventilated area.  Dispose of container in a safe manner.  For skin contact, wash with soap and water.  Soak up any residual with saw dust or other inert material.

Waste Disposal Method:  Do not discard empty container in incinerator or fire. Do not reuse empty container.  Wrap and place in trash collector.

Section VIII — Special Protection Information

Respiratory Protection(Specify Type) – Not required – avoid breathing mist.

Ventilation – Mechanical (General)  Provide adequate ventilation

Protective Gloves – Not required – avoid skin contact

Eye Protection – Not required – avoid contact with eyes

Other Protective Equipment – Use product in accordance with label

Section IXI — Special Precautions

Precautions to be taken in handling and storing – Sore in a cool dry area away from heat and open flames.  Do not store above 130 degrees F.  READ LABEL

Other Precautions – Keep out of reach of children.  Avoid contamination of food and food preparation equipment.